# CHAPTER 62-600

# DOMESTIC WASTEWATER FACILITIES

# PART I  GENERAL

# 62-600.100 	Applicability

# 62-600.120 	Exemptions

# 62-600.200 	Definitions

# PART II  TREATMENT FACILITIES

# 62-600.420 	Minimum Treatment Standards ‒ Technology Based Effluent Limitations (TBELs)

# 62-600.430 	Additional Treatment ‒ Water Quality-Based Effluent Limitations (WQBELs)

# 62-600.440 	Disinfection Requirements

# PART IV  MONITORING AND REPORTING REQUIREMENTS

# 62-600.660	Treatment Facility Monitoring

# PART V  PERMITTING

# 62-600.707 	Distributed Wastewater Treatment Systems and Distributed Wastewater Treatment Units

PART I GENERAL

## 62-600.100 Applicability.

(1) No change.

(2) The standards and requirements of Parts II and V of Chapter 62-600, and Rule 62-600.500, F.A.C., shall be applicable to septic systems and other onsite closed-tank wastewater treatment systems, including distributed wastewater treatment units (DWTUs), with subsurface disposal regulated by this chapter. The reliability requirements of paragraph 62-600.400(1)(b), F.A.C., shall not apply to such septic systems and other onsite closed-tank wastewater treatment systems with subsurface disposal. The standards and requirements of Part IV of Chapter 62-600, F.A.C., except for Rule 62-600.670, F.A.C., shall also be applicable to distributed wastewater treatment systems (DWTSs) and DWTUs with subsurface disposal.

(3) No change.

Rulemaking Authority 403.051, 403.061, 403.062, 403.086, 403.087, 403.088 FS. Law Implemented 403.021, 403.051, 403.061, 403.062, 403.085, 403.086, 403.087, 403.088 FS. History–New 11-27-89, Amended 1-30-91, Formerly 17-600.100, Amended 12-24-96, 2-8-16, xx-xx-xx.

## 62-600.120 Exemptions.

(1) Onsite sewage treatment and disposal systems are subject to the requirements of Chapter 62-6, F.A.C, and regulated by DOH are exempted from the requirements of this chapter. The Department DOH regulates onsite sewage treatment and disposal systems in accordance with Section 381.0065, F.S.

(2) No change.

Rulemaking Authority 403.051, 403.061, 403.086, 403.087, 403.088 FS. Law Implemented 403.021, 403.051, 403.061, 403.062, 403.085, 403.086, 403.087, 403.088 FS. History–New 11-27-89, Amended 1-30-91, Formerly 17-600.120, Amended 12-24-96, 2-8-16, xx-xx-xx.

## 62-600.200 Definitions.

Terms used in this chapter shall have the meanings specified below.

(1) through (19) No change.

(20) “Distributed wastewater treatment system (DWTS)” means an integrated system approach to treating wastewater consisting of one or more distributed wastewater treatment units, which is subject to applicable requirements for a sewage disposal facility pursuant to this chapter and Chapter 62-620, F.A.C.

(21) “Distributed wastewater treatment unit (DWTU)” means an advanced onsite closed-tank wastewater treatment system that is remotely operated and controlled by the permittee using an electronic control system and designed to achieve secondary treatment standards and a minimum of 80 percent total nitrogen removal before discharge to a subsurface application system.

(20) through (87) renumbered to (22) through (89)

Rulemaking Authority 403.051, 403.061, 403.086, 403.087, 403.088 FS. Law Implemented 403.021, 403.051, 403.061, 403.062, 403.085, 403.086, 403.087, 403.088 FS. History–New 11-27-89, Amended 1-30-91, 6-8-93, Formerly 17-600.200, Amended 12-24-96, 2-8-16, 9-27-21, 1-16-22, xx-xx-xx.

PART II TREATMENT FACILITIES

## 62-600.420 Minimum Treatment Standards ‒ Technology Based Effluent Limitations (TBELs).

Except as specifically required by other Department rules or Florida Statutes, all domestic wastewater facilities shall provide, at a minimum, secondary treatment of wastewater prior to reuse or disposal.  Secondary treatment requirements are specified below:

(1) through (2) No change.

(3) Reuse, land application, or groundwater discharge (including DWTUs and underground injection, but excluding potable reuse projects).

(a) through (b) No change.

(4) No change.

Rulemaking Authority 403.051, 403.061, 403.086, 403.087, 403.088 FS. Law Implemented 403.021, 403.051, 403.061, 403.062, 403.085, 403.086, 403.087, 403.088 FS. History–New 11-27-89, Amended 1-30-91, 6-8-93, Formerly 17-600.420, Amended 2-8-16, 9-27-21, xx-xx-xx.

## 62-600.430 Additional Treatment ‒ Water Quality-Based Effluent Limitations (WQBELs).

(1) No change.

(2) Ground water discharge.

(a) In addition to the TBELs specified in Rule 62-600.420, F.A.C., facilities shall provide additional treatment when needed to satisfy water quality standards for receiving ground waters. This shall not apply to DWTUs or DWTSs granted under section 403.814(13), F.S.

(b) No change.

Rulemaking Authority 403.051, 403.061, 403.086, 403.087, 403.088 FS. Law Implemented 403.021, 403.051, 403.061, 403.062, 403.085, 403.086, 403.087, 403.088 FS. History–New 11-27-89, Amended 1-30-91, Formerly 17-600.430, Amended 2-8-16, xx-xx-xx.

## 62-600.440 Disinfection Requirements.

(1) All wastewater treatment facilities shall be designed and operated to meet the disinfection requirements in this section and the microbiological pollutants criteria contained in Chapter 62-302, F.A.C. One or more DWTUs may discharge without disinfection into a slow-rate subsurface application system that meets the requirements of Rule 62-600.707, F.A.C. Where the slow-rate subsurface application system for a DWTU does not meet the horizontal setback requirements specified for a private potable well or a public drinking water well pursuant to subparagraph 62-600.707(1)(b)5., F.A.C., high level disinfection shall be required unless the applicant provides a demonstration that a reduced level of disinfection will not cause an exceedance of microbiological standards as defined in Chapter 62-550, F.A.C.

(2) through (8) No change.

Rulemaking Authority 403.051, 403.061, 403.086, 403.087, 403.088 FS. Law Implemented 403.021, 403.051, 403.061, 403.062, 403.085, 403.086, 403.087, 403.088 FS. History–New 11-27-89, Amended 1-30-91, 6-8-93, Formerly 17-600.440, Amended 12-24-96, 2-8-16, 9-27-21, xx-xx-xx.

PART IV MONITORING AND REPORTING REQUIREMENTS

## 62-600.660 Treatment Facility Monitoring.

(1) Wastewater treatment facilities shall monitor the flow, the influent for CBOD5 and TSS, and the reclaimed water or effluent for all reclaimed water or effluent parameters as required by the permit. The minimum schedule for sampling and testing parameters to be monitored at a wastewater treatment plant is specified in Figure 1 below. Monitoring requirements for DWTUs and DWTSs are specified in Rule 62-600.707, F.A.C.

Figure 1. Minimum Schedule for Sampling and Testing of Domestic Wastewater Treatment Plant Monitoring Parameters by Permitted Capacity

| Parameters | 2,000 gpd up to, but not including, 25,000 gpd | 25,000 gpd up to, but not including, 50,000 gpd | 50,000 gpd up to, but not including, 500,000 gpd | 0.5 mgd up to, but not including, 1 mgd | 1 mgd up to, but not including, 5 mgd | 5 mgd up to, but not including, 15 mgd | 15 mgd and above |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Flow, pH1 Chlorine Residual2 | daily 2, 3, or 5/wk8 | daily 5/wk | daily 5/wk | daily 5/wk | continuous | continuous | continuous |
| Dissolved Oxygen | daily 2, 3, or 5/wk8 | daily 5/wk | daily 5/wk | daily 5/wk | daily 7/wk | daily 7/wk | daily 7/wk |
| Suspended Solids4 CBOD5, Nutrients | monthly | monthly | every two weeks3 | weekly | weekly | daily 7/wk | daily 7/wk |
| Chlorine Residual5 | monthly | monthly | every two weeks3 | weekly | daily 7/wk | daily 7/wk | daily 7/wk |
| Fecal Coliform4,6 | monthly | monthly | every two weeks3 | weekly | weekly | daily 5/wk | daily 5/wk |
| Total Coliform7 | daily/7 wk | daily/7 wk | daily/7 wk | daily/7 wk | daily/7 wk | daily/7 wk | daily/7 wk |
| E. Coli9 | weekly | weekly | weekly | weekly | weekly | weekly | weekly |
| Entercocci10 | weekly | weekly | weekly | weekly | weekly | weekly | weekly |

1Hourly measurements during the period of required operator attendance may be substituted for continuous measurement.

2Total chlorine residual measured for disinfection effectiveness (after chlorine contact). Hourly measurements during the period for required operator attendance may be substituted for continuous measurement except for systems permitted under Parts III and V of Chapter 62-610, F.A.C. Continuous measurement shall be provided for all systems permitted under Parts III and V of Chapter 62-610, F.A.C., regardless of permitted capacity.

3Reuse and land application facilities (which include rapid-rate, slow-rate, absorption fields and other systems pursuant to Chapter 62-610, F.A.C.) less than 100,000 gpd, may sample monthly. (This reduction does not apply to injection wells pursuant to Chapter 62-528, F.A.C., and reuse systems requiring high-level disinfection.)

4For reuse systems requiring high-level disinfection, samples shall be obtained and reported daily, 7 days per week for systems of 0.5 mgd and greater, 4 days per week for systems of at least 50,000 gpd but less than 0.5 mgd, and 3 days per week for systems less than 50,000 gpd; or daily during the period required for operator attendance, whichever is less. At permit renewal, reduction to 4 days per week for systems of 0.5 mgd and greater or to 3 days per week for systems of at least 50,000 gpd but less than 0.5 mgd may be requested if no violations for these parameters have occurred in the last 12 months. For systems requiring high-level disinfection, the reduction allowed by note 3 does not apply.

5Total chlorine residual measured for dechlorination effectiveness.

6Not applicable to reuse systems discharging to Class I waters or waters contiguous to or tributary to Class I waters or to reuse systems injecting into Class F-I, G-I, or G-II ground waters.

7Applicable to reuse systems discharging to Class I waters or waters contiguous to or tributary to Class I waters or to reuse systems injecting into Class F-I, G-I, or G-II ground waters. These systems shall monitor 7 days per week for total coliform, regardless of permitted capacity, as specified in Parts III and V of Chapter 62-610, F.A.C.

8The daily frequency shall be 2, 3, or 5 days per week consistent with the required operator attendance specified in paragraph 62-699.310(2)(a), F.A.C.

9Applicable to surface water discharges to Class I and Class III freshwater in accordance with Rule 62-302.530, F.A.C.

10Applicable to surface water discharges to predominately marine Class III waters in accordance with Rule 62-302.530, F.A.C.

(2) through (4) No change.

Rulemaking Authority 403.061(7) FS. Law Implemented 403.061(13) FS. History‒New 2-8-16, Amended 9-27-21, xx-xx-xx.

PART V PERMITTING

## 62-600.707 Distributed Wastewater Treatment Systems and Distributed Wastewater Treatment Units.

(1) All Distributed Wastewater Treatment Units (DWTU) and Distributed Wastewater Treatment Systems (DWTS) shall meet the requirements of this section.

(a) This section applies to a DWTU that is installed and operated as part of a permitted DWTS meeting the definitions in Rule 62-600.200, F.A.C. The replacement of an existing onsite sewage treatment and disposal system (OSTDS) with a DWTU is authorized under Section 403.814(13) F.S. All other DWTSs are subject to the standard permitting provisions of this chapter and Chapter 62-620, F.A.C.

(b) General Requirements:

1. DWTUs within a DWTS shall be commonly owned, and shall be operated and maintained under the authorities of the DWTS permittee in accordance with this chapter.

2. The permittee of the DWTS shall enter into or benefit from an agreement, easement, or other legal mechanism providing for installation, operation, and maintenance of each DWTU prior to installation.

3. A permitted DWTS and its associated DWTUs shall be located within a single specified county in Florida.

4. The DWTU may discharge without disinfection, except as specified in subsection 62-600.440(1), F.A.C., into a slow-rate subsurface application system that is operated in accordance with this chapter.

5. The slow-rate subsurface application system for the DWTU shall meet the horizontal setback distance requirements set forth in Rule 62-6.005, F.A.C., or provide for disinfection in accordance with the applicable provisions of subsection 62-600.440(1), F.A.C.

6. The slow-rate subsurface application system for the DWTU shall be consistent with the site evalaution criteria of section 62-6.006, F.A.C., including the elevation of the bottom surface of the drainfield, or for replacement of an existing onsite sewage treatment and disposal system subject to regulation under section 381.0065, F.S., the DWTU installation shall maintain the current separation between the drainfield and the water table at the wettest season of the year, but in no case shall there be less than 12 inches of separation, between the bottom surface of the drainfield and the water table elevation at the wettest season of the year.

(c) Prohibitions: The operation of a DWTU and associated subsurface application system may not:

1. Create saturated conditions on the ground surface;

2. Adversely impact wetlands or other surface waters;

3. Cause or contribute to a violation of state water quality standards; or,

4. Cause a sanitary nuisance.

(d) DWTU Minimum Design Requirements: All DWTUs shall be designed to meet the following:

1. Provide, at a minimum, secondary treatment of wastewater prior to disposal as provided in subsection 62-600.420(3), F.A.C.

2. Design flow for a DWTU shall not exceed 10,000 gallons per day of domestic wastewater or 5,000 gallons per day of commercial wastewater.

3. The DWTU design and installation shall provide a means for periodic collection of representative samples of the effluent discharge from the DWTU.

4. The DWTU shall be designed to provide 80% total nitrogen removal.

(e) DWTU Minimum Treatment Requirements: If a DWTU does not meet the maximum-permissible concentration in any single sample as defined in subsection 62-600.420(3), F.A.C., the procedure in subsection 62-600.707(3), F.A.C., shall apply. Total nitrogen reduction shall be consistent with the design requirements of 80% total nitrogen removal.

(f) DWTS Minimum Treatment Requirements: To detemine DWTS compliance, DWTU sample results shall be aggregated using an arithmetic mean. Where multiple samples are collected in a single day, results for each sample must be aggregated using an arithmetic mean to provide an aggregated single sample result.

1. At a minimum, the aggregated results must meet secondary treatment requirements as specified in subsection 62-600.420(3), F.A.C.:

2. The minimum and maximum pH results across the DWTS shall meet the pH range set forth in Rule 62-600.445, F.A.C.

3. The aggregate annual average of total nitrogen removal for a DWTS shall not be less than 80%, determined as the reduction in concentration measured from influent to effluent. Total nitrogen removal shall be a rolling annual average based on aggregated monthly samples. For compliance with this provision, the influent concentration may be determined by collecting and analyzing, consistent with subsection 62-600.707(3), F.A.C., a representative sample set from permitted DWTUs for the DWTS. If influent samples results are unavailable, the permittee may use a representative Department-approved influent concentration. This aggregate annual average for total nitrogen removal shall be reported monthly on the Discharge Monitoring Report (DMR) for the DWTS.

(g) Annual Inspection: All operational DWTUs shall be physically inspected at least once annually by a technician trained by the manufacturer or professional engineer who designed or is responsible for the DWTU to evaluate the ability of the DWTU to function as intended, the physical condition and performance of each treatment component of the DWTU, the condition of the receiving subsurface application system, and any other noted problems or deficiencies.

(h) Recordkeeping Requirements. All records, notes, monitoring data, and other information relating to the installation, construction, or operation of DWTUs, or to the operation of a DWTS, shall be maintained at the site identifed for the DWTS or maintained electronically by the permittee, and shall otherwise comply with the recordkeeping requirements specified under Rule 62-620.350, F.A.C. The following records shall be maintained and available for inspection by the Department:

1. Records of all compliance monitoring information, including all calibration and maintenance records and all original strip chart recordings for continuous monitoring instrumentation, including, if applicable, a copy of the laboratory certification showing the certification number of the laboratory, for at least three years from the date the sample or measurement was taken.

2. Copies of all reports, other than those required in subparagraph 1., 4., and 6., of this paragraph, are required to be retained for at least three years from the date the report was prepared;

3. Records of all data, including reports and documents, used to complete the application for the permit for at least three years from the date the application was filed;

4. Monitoring information, including a copy of the laboratory certification showing the laboratory certification number, related to the residuals use and disposal activities for the time period set forth in Chapter 62-640, F.A.C., or for at least three years from the date of sampling or measurement;

5. Copies of the licenses of the current certified operators; and,

6. Copies of the logs and schedules showing DWTU operations and equipment maintenance for three years from the date on the logs or schedules. Operation and maintenance log data must be accessible upon request and protected from weather damage. If kept electronically, operator must ensure adequate data storage capacity and data backup. Entries made by recording equipment shall be date/time stamped. Entries made by an operator shall be date/time stamped and accompanied by an electronic signature unique to, and under the sole control of, the operator. Logs shall reflect the initial date a DWTU is placed into operation.

(i) All DWTS permittees shall be responsible for providing notice to the appropriate Department district office within 72 hours of discovery of any instances detailed below in subparagraph 1. through 4. The notice shall include information on the quality and quantity of influent introduced into the DWTU and any anticipated impact on the quality or quantity of effluent discharging from the DWTU.

1. Any changes to a DWTU’s average daily flow outside of the DWTU’s maximum design flow, any introduction of toxic, hazardous, or industrial waste to a DWTU, any change from domestic wastewater to commercial wastewater to a DWTU, or any other changes in the wastewater flow to a DWTU that would be expected to adversely affect the treatment performance of the DWTU;

2. Any instances where a DWTU has failed from reasons other than loss of power for periods in excess of 72 hours;

3. Any instances of repairs or emergency maintenance that resulted in a DWTU being offline for more than 72 hours; and,

4. Any instances when the prohibitions in subparagrah (c) above occur or any other instances of non-compliance with the requirements of Rule 62-600.707, F.A.C.

(j) Biosolids management. Biosolids, or other materials pumped from DWTUs, shall be either transferred to a biosolids treatment facility in accordance with Chapter 62-640, F.A.C., or handled as septage. Hauling records, for any biosolids or other materials pumped from a DWTU, shall be maintained for a minimum of five years.

(2) DWTS and DWTU Notification and Certification Requirements:

(a) The notification as required by subsection 403.814(13)(b), F.S., must be submitted on DEP Form 62-600.707(2)(a), Distributed Wastewater Treatment Unit (DWTU) General Permit Notification, effective [proposed effective date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXX, hereby adopted and incorporated by reference, and must be received at least 30 days prior to installation. All certifications shall include the following:

1. Identification of the DWTU using a unique indentifying number, latitude and longitude, property address, and site plan;

2. The minimum and maximum design flows based on the specific application and the design capacity of the DWTU;

3. An assessment of the quality of influent introduced into a commercial DWTU, if applicable;

4. Monthly Effluent Compliance Sampling Plan; and

5. Proof of legal access and customer agreement.

(b) The permittee for a DWTS, either granted under subsection 403.814(13)(b), F.S., or under a permit issued pursuant to Chapter 62-620, F.A.C. and this chapter, shall provide a certification of completion of construction, within 30 days following installation of a DWTU, on DEP Form 62-600.707(2)(b), Notification of Completion of Construction for Distributed Wastewater Treatment Unit (DWTU). effective [proposed effective date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXX, hereby adopted and incorporated by reference. The certification of completion of construction shall include the following:

1. A dimensioned as-built sketch of the DWTU and subsurface absorption field with measurements from fixed structures, lot lines, and items subject to horizontal separation requirements; and

2. Soil and groundwater analysis sufficient to confirm the slow-rate subsurface application system design shall be consistent with the applicable requirements under Rule 62-6.008, F.A.C.

(c) The Department shall transfer a general permit granted under subsection 403.814(13), F.S., following notification of such transfer where a “Notification of Transfer of a DWTS or DWTU,” DEP Form 62-600.707(2)(c), effective [PROPOSED EFFECTIVE DATE], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXX, hereby adopted and incorporated by reference., is submitted to the appropriate district office within 30 days of legal transfer of ownership of a DWTS or DWTU. Until the permit is transferred pursuant to this paragraph, the existing permittee shall remain liable for compliance with the terms of the applicabe DWTS permit.

(d) To terminate coverage of a DWTU under a general permit or a permit issued under Chpater 62-620, F.A.C., the permittee shall submit “Notice of Termination of Distributed Wastewater Treatment Unit (DWTU),” DEP Form 62-600.707(2)(d), effective [PROPOSED EFFECTIVE DATE], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXX, hereby adopted and incorporated by reference, to the appropriate district office within 30 days after abandonment and terminating operation of the DWTU.

(e) All certifications and accompanying documentation shall be submitted online through the DEP Business Portal at http://www.fldepportal.com/go/.

(3) Requirements for an Effluent Compliance Sampling Plan.

(a) Effluent Compliance Sampling Plan: All operating DWTS shall comply with an applicable Effluent Compliance Sampling Plan that meets the requirements of this subsection. For a DWTS authorized pursuant to the general permit granted under section 403.814(13), F.S., an Effluent Compliance Sampling Plan meeting the requirements of this subsection shall be submitted to the Department prior to operation of the first DWTU within the corresponding DWTS. For a DWTS authorized under a permit issued pursuant to Chapter 62-620, F.A.C., and this chapter, a proposed Effluent Compliance Sampling Plan shall be submitted along with the permit application for the proposed DWTS. An Effluent Compliance Sampling Plan under this subsection shall, at a minimum, meet the following requirements:

1. Effluent data shall be submitted to the Department on a monthly basis and in accordance with Chapter 62-620, F.A.C.

2. DWTSs shall monitor the flow, CBOD5, TSS, pH, and total nitrogen of the effluent in accordance with the treatment requirements as defined in paragraph 62-600.707(1)(d) through (f), F.A.C.

2. Sampling and monitoring data shall be collected and analyzed in accordance with Rule 62-4.246, Chapters 62-160 and 62-620, F.A.C., and 40 C.F.R. 136, as appropriate.

3. If the permittee monitors any contaminant more frequently than required by the permit, using Department approved test procedures, the results of this monitoring shall be included in the calculation and reporting of the data submitted in the DMR.

4. Calculations for all monitoring results which require averaging of measurements shall use an arithmetic mean.

5. Except as specifically provided in Rule 62-160.300, F.A.C., any laboratory test required shall be performed by a laboratory that has been certified by the Department of Health Environmental Laboratory Certification Program (DOH ELCP). Such certification shall be for the matrix, test method and analyte(s) being measured to comply with the requirements set forth in rule 62-600.707, F.A.C. For DWTUs, testing for parameters listed in subsection 62-160.300(4), F.A.C., shall be conducted under the direction of a certified operator.

6. Field activities including onsite tests and sample collection shall follow the applicable standard operating procedures described in DEP-SOP-001/01 adopted by reference in Chapter 62-160, F.A.C.

7. Alternate field procedures and laboratory methods may be used where they have been approved in accordance with Rules 62-160.220 and 62-160.330, F.A.C.

8. A minimum of five percent of the DWTUs within a DWTS must be sampled for compliance monthly. DWTU selection for compliance sampling shall be randomized and representative of the system. Each DWTU shall be sampled for compliance at least once every two years, which may include the results of monthly compliance sampling as required herein. For DWTS that are comprised of more than 400 DWTUs, the applicant may propose an alternative sampling plan, and the Department may approve an alternative sampling plan that provides an equivalent statistically representative sample size for the operational DWTUs.

9. All newly installed DWTUs shall be sampled within six months of being placed into operation and after the DWTU has had a minimum of 60 days of operation.

10. Compliance sampling is not required on DWTUs receiving less than their design minimum flow over the last 30 days.

11. For any individual DWTU that exceeds the maximum-permissible concentration in any single sample as defined in subsection 62-600.420(3), F.A.C., the DWTU shall be resampled monthly for the exceeding parameter until compliance limits are met, after which the minimum sampling frequency of the DWTU may resume.

12. When a DWTU exceeds the maximum-permissible concentration in any single sample as defined in subsection 62-600.420(3), F.A.C. for three consecutive monthly sampling periods, a remediation plan shall be included with the monthly DMRs, and shall be implemented by the DWTS permittee.

(b) Reporting Requirements: In accordance with Rule 62-600.680, F.A.C., the permittee must report monthly aggregate monitoring results on a Discharge Monitoring Report (DMR) for the DWTS, DEP Form 62-600.707(2)(d), for all parameters as identified for the Effluent Compliance Sampling Plan under subsection 62-600.707(3), F.A.C. Sampling results from individual DWTUs shall be maintained for inspection, if requested by the Department, in accordance with the record keeping requirements specified under Rule 62-620.350, F.A.C. The permittee shall use the DMR provided to the permittee by the Department in accordance with a permit issued pursuant to Chapter 62-620, F.A.C., and this chapter, or following the notification submitted in accordance with 403.814(13)(b), F.S.

(c) In accordance with subsection 62-600.650(3), F.A.C., after five years of sampling results showing the DWTU is in compliance with all required performance standards, a permittee may submit a written request to the Department for a reduction in monitoring requirements. The request shall include a summary of the data for all tests being considered. Reductions in monitoring shall only become effective upon the Department’s written confirmation.

Rulemaking Authority 403.061, 403.087, 403.088, 403.814, 403.88(4) FS. Law Implemented 403.061, 403.087, 403.0877, 403.088, 403.814 FS. History–New xx-xx-xx.