# CHAPTER 62-699

# TREATMENT PLANT CLASSIFICATION AND STAFFING

# 62-699.200 	Definitions.

# 62-699.310 	Classification and Staffing of Domestic Wastewater or Water Treatment Plants and Water Distribution Systems

# 62-699.311 	Additional Classification and Staffing Requirements

## 62-699.200 Definitions.

For the purposes of this chapter, the following words, phrases, or terms shall have the following meaning.

- through (6) No change.

(7) “DISTRIBUTUTED WASTEWATER TREATMENT SYSTEM” or “DWTS” means the same as defined in subsection 62-600.200(20), F.A.C.

(8) “DISTRIBUTUTED WASTEWATER TREATMENT UNIT” or “DWTU” means the same as defined in subsection 62-600.200(21), F.A.C.

(9) “ELECTRONIC CONTROL SYSTEM” means a comprehensive automatic control system plus electronic surveillance system along with capability for manual adjustment and control of domestic wastewater or water treatment plant equipment and processes via a computerized system at a central or off-site location that is staffed in accordance with this Chapter. by a Class C or higher treatment plant operator 24 hours per day and seven days per week.

(8) through (9) Renumbered as (10) through (11).

(12) “ONSITE,” when referring to operating, staffing, visiting, or checking a domestic wastewater or water treatment plant, means at the location of the plant; when referring to operating, staffing, or checking a DWTU means to actively control and operate the DWTU using an electronic control system; and when referring to operating or staffing a water distribution system, means at the location of the distribution system pipe, pumps, or tanks where operation or maintenance activities that may affect water quality or quantity are taking place.

(11) through (16) Renumbered as (13) through (18).

Rulemaking Authority 403.869, 403.88(4) FS. Law Implemented 403.182, 403.852, 403.862, 403.866 FS. History–New 10-15-07, Amended 3-6-13, xx-xx-xx.

## 62-699.310 Classification and Staffing of Domestic Wastewater or Water Treatment Plants and Water Distribution Systems.

This section applies to all domestic wastewater treatment plant permittees, which includes DWTS permittees, and to all suppliers of water except those who own or operate a transient non-community water system (TWS) using only ground water as a source of supply and serving only religious institutions, or businesses, other than public food service establishments as defined and regulated under Chapter 381, 500, or 509, F.S., and other than religious institutions with school or daycare services. Licensed operators are not required for TWSs using only ground water as a source of supply and serving only religious institutions, or businesses, other than public food service establishments and other than religious institutions with school or daycare services.

(1) No change.

(2) Permittees and suppliers of water shall employ licensed operators onsite at their domestic wastewater treatment plant(s), DWTS(s), or water treatment plant(s) to fulfill the time and visit requirements specified in paragraphs (a) through (e), below. Suppliers of water shall employ licensed operators to staff their water distribution system as specified in paragraph (f), below.

(a) Classification and Staffing Requirements for Domestic Wastewater Treatment Plants Other than Those Discussed Separately in paragraphs (b) through (d), below. Refer to subparagraphs tables 1. through 5. 4., below, and determine the classification and staffing requirements for each domestic wastewater treatment plant using the following two-step procedure: first determine the category of the plant and then, within that category, determine the classification and staffing requirements for the plant. Determine the plant category by identifying the highest category in subparagraphs tables 1. through 5. 4., listing one or more of the plant’s treatment processes, with Category I being the highest category and Category IV being the lowest category. Determine the plant classification and staffing requirements within the determined plant category by using the permitted capacity of the plant. Class A is the highest possible plant classification, and Class D is the lowest possible plant classification.

1. through 4. No change.

5. DWTS requires an appropriately licensed wastewater treatment plant operator, Class C or higher, to monitor and evaluate, via electronic control system, the performance of each unit covered under the DWTS permit. Monitoring shall be performed at least once during a weekday and once during the weekend.

a. All maintenance, whether preventative or required to return the system to compliance, shall be performed by an appropriately licensed operator in-person at the physical location of the DWTU unless the issue can be resolved via the electronic control system. When maintenance must be performed at the physical location of the unit due to conditions not in accordance with the requirements of paragraph 62-600.707(1)(c), F.A.C., it shall be responded to within 24 hours. Maintenance at the physical location of the unit may also be performed by an operator’s unlicensed designee while under the off-site supervision of the licensed operator, meaning the licensed operator directs, controls, and is accountable for maintenance activities to ensure compliance with applicable laws, regulations, permit conditions, and accepted professional practices.

b. Experience obtained in the operation of a DWTS can only be applied towards experience outlined in paragraph 62-602.250(1)(b), F.A.C., in pursuit of obtaining a Class C or D wastewater treatment plant operator license pursuant to 62-602, F.A.C.

(b) through (f) No change.

Rulemaking Authority 403.869, 403.88(4) FS. Law Implemented 403.853(6), 403.867, 403.875(1)(f), 403.88(1), (2), (3) FS. History–New 11-17-70, Revised 10-24-74, Amended 12-25-75, 6-10-76, 1-12-81, Formerly 17-16.12, 17-16.13, Amended 5-8-85, Formerly 17-16.370, 17-602.370, 17-699.310, Amended 10-15-07, 3-6-13, xx-xx-xx.

## 62-699.311 Additional Classification and Staffing Requirements.

(1) No change.

(2) A daily check of each Class C or D domestic wastewater treatment plant shall be performed by the permittee, or his or her representative or agent, each day the plant is in operation or 5 days per week, whichever is less. The daily check of a plant shall include on-site observation of the plant to verify that it is functioning and may include the taking of readings or samples or the performing of tests or measurements; and the daily check of a plant shall be noted in the operation and maintenance log required under subsection 62-602.650(4), F.A.C. A daily visit, or daily staffing, of a plant by an appropriately licensed operator fulfills the requirement for a daily check of the plant. Nothing in this subsection (2), shall be construed as allowing anyone to perform compliance tests for domestic wastewater unless he or she meets applicable qualification requirements in subsection 62-601.400(3), F.A.C. For DWTS, daily checks may be accomplished utilizing an electronic control system.

(3) through (15) No change.

Rulemaking Authority 403.869, 403.88(4) FS. Law Implemented 403.867, 403.88(1), (2), (3) FS. History–New 11-17-70, Revised 10-24-74, Amended 12-25-75, 6-10-76, Formerly 17-16.13, Amended 5-8-85, Formerly 17-16.375, 17-602.375, 17-699.311, Amended 7-5-01, 10-15-07, 3-6-13, xx-xx-xx.