62-330.601 General Permit for the Installation, Construction, and Maintenance of Inshore Reefs

(1) A general permit is hereby provided for the installation, placement, restoration, and enhancement of an inshore reef within estuaries, lagoons, or other near shore tidal waters, subject to the following:

(a) The reef project shall be for habitat enhancement purposes and shall not be used for mitigation for any other project, or be associated with mitigation banking or water quality enhancement areas;

(b) The reef shall be constructed of the following materials:

1. Natural oyster or clam shell cultch;

2. Oyster or clam shells packaged within bags that are biodegradable in salt water or bags that are free from petroleum or plastic;

3. Clean concrete, bio-enhancing concrete, or Florida limestone rock with a thickness of 1/4 inch or greater; or

4. Prefabricated modules of appropriate size that are a mixture of clean concrete, bio-enhancing concrete, rock, or fossilized or cured shells. Prefabricated modules may include rebar as structural support if completely encased in concrete;

a. For the purpose of this general permit, clean concrete is defined as concrete material that is free of contaminants, coatings, reinforcing steel, and other hazardous substances;

b. For the purpose of this general permit, bio-enhancing concrete is defined as concrete formulated with bio-enhancing or bioactive admixtures that promote marine biotic growth.

(c) The constructed reef shall not contain soils, oils and greases, steel, exposed rebar, debris, litter, plastic, asphalt material, tires, fiberglass, putrescible substances or other pollutants;

(d) The total cumulative footprint of the reef shall not exceed 2,500 square feet;

(e) The reef shall not adversely affect submerged grassbed communities, shellfish or other hardbottom communities, or corals;

(f) The reef materials shall be firmly affixed to a structure or the substrate or otherwise contained in such a way as to prevent movement away from the authorized footprint.

(2) The notice required in Rule 62-330.402, F.A.C., shall include documentation, statements, or demonstrations that the above limitations will be met, together with the following:

(a) An explanation of the overall purpose and any expected ecological benefits to be achieved by the reef;

(b) Written authorization from the owner(s) of the submerged lands, if other than the State of Florida, for the permittee to conduct the proposed activities; and

(c) An environmental resource survey conducted during seagrass growing season of the bottom of the waterbody throughout the reef area, including an additional 50-foot-wide perimeter surrounding the proposed footprint of the reef area demonstrating that:

1. There are no submerged grassbed communities, shellfish or other hardbottom communities, or corals within the footprint of the constructed reef. Co-mingling of reefs within other restored, created, or enhanced habitats can be considered;

2. The reef shall not adversely affect an archaelogical, cultural, or historical resource area regulated under Chapter 267, F.S.

(3) All work conducted under this general permit shall be subject to the following specific conditions:

(a) If shell material is to be used within the reef it shall consist of only the following:

1. Recycled shell that has been quarantined for a minimum of three months; or

2. Fossil shell that must not contain any phosphate, other nutrients, or radiation.

(b) The reef site shall not be constructed within:

1. 328 feet of actively nesting wading bird rookeries and seabird colonies or within 590 feet of actively nesting Least Tern or Black Skimmer colonies, or within 328 feet of actively nesting American Oystercatchers or Snowy Plovers, or other state listed solitary shorebirds;

2. 150 feet from the outside edge of any designated channel marked in accordance with Section 327.40, F.S.;

3. Any federally maintained channel without written approval from the entity responsible for channel maintenance; or

4. Any channel traditionally used for navigation;

5. Sandy beaches, shorelines, or coastal waters and associated inlets or passes in which activities are regulated under Chapter 161, F.S., or the open waters of the Atlantic Ocean, Gulf of America, or Straights of Florida;

6. The open waters of the Atlantic Ocean or the Gulf of America.

(c) The reef material shall be placed within the authorized footprint and not be indiscriminately dumped;

(d) Reef units must be placed so that there is at least one opening measuring at least 5 feet in width every 75 linear feet along the structures, with a minimum of one opening, to allow the flow of water and the passage of fishes and aquatic wildlife;

(e) Any stabilization must be designed to avoid the entanglement of marine life;

(f) The reef shall be designed to avoid harmful erosion or shoaling to adjacent submerged areas and shorelines; and,

(g) The Permittee shall notify the Department upon completion of the inshore reef project and shall provide a monitoring report within one year following completion. The report shall include the following:

1. The permit number;

2. Color photographs that provide an accurate representation of the project site;

3. A map showing the location of all installed inshore reef structures including GPS coordinates;

4. A brief written report detailing the status of the project, the condition of all structures, and the accumulation of microorganisms, plants, algae, or animals on the reef structures;

5. Any other relevant information that indicates the possibility of disturbances to the inshore reef, such as, but not limited to, fishery and wildlife resource observations, significant nearby features, or site disturbances.

(h) The Permittee shall be responsible for the maintenance of the reef including the retrieval, removal and proper disposal of any adrift, derelict, or dilapidated reefs or reef materials; and

(i) Adrift, derelict, or dilapidated materials or structures shall be removed within 30 days upon notification from the Department.

Rulemaking Authority 373.043, 373.118(1), 373.406(5), 373.4131, 373.414(9), 373.418, 403.805(1) FS. Law Implemented 373.118(1), 373.406(5), 373.413, 373.4131, 373.414(9), 373.416, 373.418, 403.814(1) FS. History–New.