62-330.604 General Permit for the Installation, Construction, and Maintenance of Living Shorelines

(1) Living shorelines are defined as a shoreline management practice that protects, restores, and enhances natural shoreline habitat; provides erosion control benefits; and maintains riverine or coastal processes through the strategic placement of native plants, and, if necessary, native rock, prefabricated concrete modules, untreated native wood, or other structural organic materials.

(2) A general permit is hereby provided for the construction, installation, and maintenance, including repair and removal, of a living shoreline project, subject to the following:

(a) The project shall be for shoreline restoration, enhancement, or stabilization purposes and shall not be considered as mitigation for any other project or be associated with mitigation banking or water quality enhancement areas;

(b) Plantings shall consist of native emergent plants, including mangroves, or wetland vegetation obtained from commercially or nursery grown stock;

(c) Plantings shall not consist of submerged aquatic vegetation;

(d) If permanent wave attenuation is required to support shoreline vegetation, a wave attenuating structure in the form of a marsh sill is authorized to be constructed with the plantings, provided that:

1. The marsh sill shall be composed predominantly of natural oyster shell cultch contained in bags that are biodegradable in salt water or bags that are free from petroleum or plastic, clean or bio-enhancing concrete or Florida limestone rock, untreated native wood, biodegradable fiber logs or mats, or prefabricated modules that are a mixture of clean or bio-enhancing concrete, native rock, fossilized shells, or cured shells. Prefabricated modules may include rebar as structural support if completely encased in concrete;

a. For the purpose of this general permit, clean concrete is defined as concrete material that is free of contaminants, coatings, reinforcing steel, and other hazardous substances;

b. For the purpose of this general permit, bio-enhancing concrete is defined as concrete formulated with bio-enhancing or bioactive admixtures that promote marine biotic growth;

2. The material used to construct the marsh sill must be of appropriate size for the location’s wave energy and shall be firmly affixed to the substrate, or otherwise contained in such a way as to prevent movement away from the authorized footprint;

3. The marsh sill shall not contain soils, oils and greases, steel, exposed rebar, debris, litter, plastic, asphalt materials, tires, fiberglass, putrescible substances, or other pollutants; and,

4. The marsh sill shall be placed in units so that there is at least one opening measuring at least 5 feet in width every 75 linear feet along the structure, with a minimum of one opening, to allow the flow of water and the passage of fishes and aquatic wildlife.

(e) Regrading or recontouring of the submerged bottom with the addition of outside fill is not authorized under this general permit.

(3) The notice required in Rule 62-330.402, F.A.C., shall include documentation, statements, or demonstrations that the above limitations will be met, together with the following:

(a) Color photographs that provide an accurate representation of the project sites;

(b) A narrative description regarding the current condition of the shoreline; and,

(c) Scaled or dimensioned project plans, including construction methodology, showing the entire project area and all proposed activities within said area;

(d) An explanation of the overall purpose and any expected ecological benefits to be achieved by the project;

(e) Written authorization from the owner(s) of the submerged lands, if other than the State of Florida, for the permittee to conduct the proposed activities; and,

(f) An environmental resource survey of the shoreline and bottom of the waterbody throughout the project area, including an additional 50-foot-wide perimeter surrounding the project area demonstrating:

1. The existence of any emergent or submerged natural resources or exotic vegetation along the shoreline or within the proposed footprint of the living shoreline; and,

2. The methods of planting and marsh sill installation so as not to adversely affect existing emergent or submerged natural resources.

(4) All work under this general permit shall be conducted in conformance with the following specific conditions:

(a) All equipment used during construction shall be staged and stored appropriately, so as not to adversely affect the ecology of existing emergent or submerged natural resources;

(b) The project shall not adversely affect an archaeological, cultural, or historical resource area regulated under Chapter 267, F.S.;

(c) The length of the project shall not exceed 2000 linear feet along the parcel’s shoreline;

(d) The most waterward feature shall not extend more than 30 feet waterward of the mean high water line (MHWL) or ordinary high water line (OHWL);

(e) For the purpose of this general permit, a marsh sill shall:

1. Be constructed parallel to the plantings on the waterward side;

2. Not impede navigation or create a navigational hazard;

3. Not be placed within 3 feet of an area greater than 1% coverage by existing emergent or submerged natural resources.

(f) The Permittee shall notify the Department upon completion of planting and shall provide a brief monitoring report 30 days following initial planting and after the second year of establishment of vegetation. The monitoring shall be conducted from June through September of each year. The report shall include the following:

1. The permit number;

2. Color photographs that provide an accurate representation of the project; and

3. A brief statement concerning the status of the project, including the survivorship of the plantings and any other relevant information.

(g) The Permittee shall be responsible for the maintenance of the living shoreline, including the retrieval, removal and proper disposal of any adrift, derelict, or dilapidated materials or structures; and

(h) Adrift, derelict, or dilapidated materials or structures shall be removed within 30 days upon notification from the Department.

Rulemaking Authority 373.043, 373.118(1), 373.406(5), 373.4131, 373.414(9), 373.418, 403.805(1) FS. Law Implemented 373.118(1), 373.406(5), 373.413, 373.4131, 373.414(9), 373.416, 373.418, 403.814(1) FS. History–New.