CHAPTER 62-308

SYNTHETIC TURF

62.308.100     Synthetic Turf

62-308.100 Synthetic Turf

(1) The minimum standards provided in this rule only apply to the installation of synthetic turf, as defined by section 125.572(1), F.S., on single-family residential properties of 1 acre or less in size.  Any ordinance, resolution, order, rule or policy adopted by a local government pertaining to the installation of synthetic turf must be consistent with this rule. Nothing in this rule prohibits a local government from the following:

(a) Requiring documentation of compliance, including but not limited to, a permit or inspections associated with the installation of synthetic turf in accordance with this rule.

(b) Restricting synthetic turf on rights-of-way owned or giving legal right or control to an entity other than the homeowner.

(2) Material type.

(a) Synthetic turf, including backing material and infill, must not contain heavy metals or intentionally added per- and polyfluoroalkyl substances.

(b) Synthetic turf, including backing materials and infill, must be disposable under normal conditions at any Chapter 62-701, F.A.C., Florida permitted landfill.

(c) Infill material, if used, must be clean silica sand, rock, shell, or other natural material. Rubber or any other synthetic infill material is not permitted. Installation shall be designed to prevent washing away of any infill material off the residential property.

(d) Subgrade shall be composed of natural materials, such as crushed rock, that meets the permeability requirements of this rule.

(3) Color. No local government may prohibit the use of green synthetic turf.

(4) Permeability.

(a) Synthetic turf must be permeable and affixed to permeable backing with a pervious subgrade.

(b) Synthetic turf must be installed over a subgrade prepared for positive drainage and evenly graded porous material.

(5) Stormwater management.

(a) Installation of synthetic turf must be designed and installed to prevent pooling or an increase in the stormwater runoff volume, direction, or rates to adjacent properties and, where possible, runoff shall be directed to on-site pervious areas.

(b) Installation of synthetic turf must not alter the permitted stormwater management system as designed and shall not be installed within a swale, ditch, stormwater pond, or a stormwater pond’s littoral zone.

(6) Potable water conservation.

(a) In-ground irrigation systems cannot be used to irrigate synthetic turf areas.

(b) If any in-ground system is already installed, a local government may require that irrigation heads be removed and pipe capped.

(7) Water quality.

(a) Synthetic turf shall not cause or contribute to violations of state water quality standards.

(b) Buffer zones around natural or man-made waterbodies may be established to protect against erosion and reduce pollution provided that such buffer for synthetic turf is no greater or restrictive than what is applicable to natural turf. Where no buffer zone has been established, synthetic turf shall be installed no closer than 10 feet from a natural or man-made waterbody as measured from the applicable ordinary or mean high water line except where there is a physical barrier between the synthetic turf and the waterbody (such as, but not limited to, a seawall or bulkhead).

(8) Proximity to trees and other vegetation.

(a) Installation of synthetic turf cannot compromise the health of non-invasive nearby trees (which may include damage to tree roots) and vegetation.

(b) Synthetic turf shall not be installed inside the non-invasive tree drip lines, whether on the property or adjacent properties unless a certified arborist certifies that installation within that drip line would not be harmful to the tree.

(9) Other factors impacting environmental conditions of adjacent properties.

(a) Synthetic turf shall be installed according to manufacturer’s specifications.

(b) Synthetic turf shall be anchored at all edges and seams to ensure that turf will withstand the effects of wind or flooding.

(c) If installed, synthetic turf must provide for access to the septic tank for routine pumpout.

(d) If installed, synthetic turf shall be installed landward of any dune system and shall not be used to replace any existing dune vegetation.

Rulemaking Authority 125.572 FS. Law Implemented 125.572 FS. History–New xx-xx-xxxx.