# TIMELINE OF ACTIVITIES

Former Indian River State College

| DATE | ACTIONS | Program or Agency Involved | NOTES |
| --- | --- | --- | --- |
| 1/6/20 | Initial request by SIS to access property. Site access granted. | DEP- Board of Trustees of IRSC | DEP secured site access through email chain and signed access agreement form DEP site number: ERIC_7538. |
| 1/6/20 | DEP Central Laboratory certified DOH private supply well results | DOH | DWASTEWELL-2019-12-12-04. Ten supply well results. Four results were above the Health Advisory Level (HAL) for combined perfluorooctanoate acid (PFOA) and perfluorooctane sulfonate (PFOS). |
| 3/10/20 | DEP Central Laboratory certified DOH private supply well results | DOH | DWASTEWELL-2020-02-14-01. Ten supply well results. One result was above the Health Advisory Level (HAL) for combined perfluorooctanoate acid (PFOA) and perfluorooctane sulfonate (PFOS). |
| 3/27/20 | Trip Report – Monitoring Well Installation and Site Assessment (Soil and Upper Surficial Groundwater) | DEP- State Contractor | State Contractor conducted activities from 3/23/20 through3/26/20. Activities included: -Observed a private utility locate to identify any potential subsurface utilities or obstructions. -Completed 20 soil borings to 10 feet (ft) below land surface (BLS). Borings were hand augered from 0 to 5 ft BLS and completed via direct push technology (DPT) from 5 to 10 ft BLS. The lithology was described at each boring and discrete soil samples were collected. -Collected six surface water samples. -Collected six hand-augered sediment samples from 0 to 1 ft BLS. -Observed the completion of one DPT groundwater boring to refusal at 90 ft BLS; groundwater samples were collected using a 4-ft DPT screen point sampler at 10 ft intervals from 16 ft BLS until refusal using high density polyethylene tubing and a check ball valve. -Observed the installation of six monitoring wells screened from 5 to 15 ft BLS via hollow stem auger. -Collected one composite soil investigation derived waste (IDW) sample and three water IDW samples. |
| 4/23/20 | Certified sample event results received from FDEP Central Laboratory. | DEP, DEP Central Laboratory, DOH | Concentrations of perfluorooctanoate acid (PFOA) and perfluorooctane sulfonate (PFOS) in groundwater were above the Health Advisory Level (HAL) for combined (PFOA) and (PFOS) and concentrations of PFOA and PFOS in soil were above provisional soil cleanup target levels. Results sent to DOH, appropriate DEP district and local staff. DOH requested to conduct nearby potable supply well sampling. |
| 5/15/20 | Trip Report – Monitoring Well Installation and Site Assessment (Upper and Lower Surficial Aquifer Groundwater) | DEP- State Contractor | State Contractor conducted activities from 4/29/20 through 5/14/20. Activities Included: -Established staging areas at FIRSC for equipment, materials, and investigation-derived waste; -Observed the construction of the decontamination pit that was used throughout the well installation; -Observed private utility locates to identify potential subsurface utilities or obstructions on 29 April and 4 May 2020; -Observed lane-closure maintenance of traffic implementation during field activities conducted in St. Lucie County rights-of-way; -Observed the installation of 6 monitoring wells (DEPMW-7 through DEPMW-12) completed from 5 to 15 ft BLS for the investigation of PFAS-impacted groundwater in the upper surficial aquifer associated with FIRSC; -Observed the installation of 9 monitoring wells (DEPMW-13 through DEPMW-21) for the investigation of PFAS-impacted groundwater in the lower surficial aquifer associated with FIRSC (1 well completed from 80 to 100 ft BLS, 1 well completed from 85 to 105 ft BLS, and 7 wells completed from 90 to 110 ft BLS; monitoring well completion intervals were adjusted based on field lithological observations to target productive sand units); -Collected core samples for lithologic description at the well locations; and -Collected two equipment blanks (EQB-7 and EQB-8) and one field reagent blank (FRB-5). |
| 5/20/20 | Trip Report – Groundwater Sampling and Monitoring Well Survey | DEP- State Contractor | State Contractor conducted activities from 5/18/20 through 5/19/20. Activities Included: -Collected depth-to-groundwater measurements from 21 monitoring wells (DEPMW-1 through DEPMW-21) -Sampled 21 monitoring wells (DEPMW-1 through DEPMW-21) including collection of duplicate samples at DEPMW-3 and DEPMW-13; -Collected one equipment blank sample (EQB-9) from the tubing used during groundwater sampling; -Collected one field reagent blank (FRB-6); and -Observed the top-of-casing (TOC) elevation and horizontal survey for 21 newly installed monitoring wells. |
| 6/03/20 | Certified sample event results received from FDEP Central Laboratory. | DEP, DEP Central Laboratory, DOH | Concentrations of perfluorooctanoate acid (PFOA) and perfluorooctane sulfonate (PFOS) in groundwater were above the Health Advisory Level (HAL). Results sent to DOH, appropriate DEP district and local staff. DOH requested to conduct nearby potable supply well sampling. |
| 6/19/20 | Site Assessment Report (SAR) | State Contractor | Between March and May 2020, State Contractor completed a Site Assessment. |
| 8/11/20 | DEP Central Laboratory certified DOH private supply well results | DOH | DWASTEWELL-2020-07-29-01. Six supply well results. One result was above the Health Advisory Level (HAL) for combined perfluorooctanoate acid (PFOA) and perfluorooctane sulfonate (PFOS). |
| 8/28/20 | Off-Site Noticing | DEP- State Contractor | Off-Site Noticing letters mailed out to individual property owners who have suspected groundwater contamination. |
| 9/16/20 | DEP Central Laboratory certified DOH private supply well results | DOH | DWRMWELL-2020-08-21-08. Seven nearby supply well sample results. Four results were above the HAL for combined PFOA and PFOS. |
| 9/23/20 | DEP Central Laboratory certified DOH private supply well results | DOH | DWRMWELL-2020-09-16-01. Nine nearby supply well sample results. One result was above the HAL for combined PFOA and PFOS. |
| 2/15/25 | TA, Notice of Contamination Beyond Property Boundaries Executed | State Contractor | Offsite notification package including preparation of contamination notification data table and notice of contamination beyond property boundary forms is being prepared for 165 parcels surrounding FIRSC. |
| 11/7/2025 | TA. Notice of Contamination Beyond Property Boundaries Completed | DBSP | Notices were sent out to stakeholder 10/2025 |
| 11/7/2025 | Further Assessment | DEP-Contractor | New task assignment executed on 9/23 with further assessment to continue into 2026. |