# TIMELINE OF ACTIVITIES

Orange County Public Schools Education Complex Training Facility

| DATE | ACTIONS | Program or Agency Involved | NOTES |
| --- | --- | --- | --- |
| 12/6/22 | Site access granted. | DEP- School Board Chair of OCPS | DEP secured site access through email chain and signed access agreement for DEP site number: ERIC_17862 |
| 2/12/25 | Site Assessment Cost Proposal Submitted and Under DEP Review | DEP – Terracon – OCPS and Valencia College Staff | Cost Proposal for Soil, Sediment, Water Sampling, and installation of ~18 shallow and ~6 deep monitoring wells culminating in an SSAR has been submitted and is under review. |
| 11/7/2025 | Site Assessment Completed | DEP - Terracon | Current phase of site assessment has concluded with completion of SSAR on 10/17/2025. Further assessment will continue in 2026. |
| 5/25/21 | Proposed workplan for Supplemental Site Activities | DEP and State Contractor | Proposed Sample Collection to Include: 6 surface water samples 1 duplicate surface water sample 145 groundwater samples (116 DPT and 29 MW) 14 duplicate groundwater samples (11 DPT and 3 MW) 1 equipment blank for surface water sampling 7 equipment blanks for DPT GW sampling 2 equipment blanks for MW installation and sampling 4 field blanks for the various water media 1 extra field blank DPT groundwater and surface water sampling is tentatively scheduled to begin the week of July 12th. Monitoring well installation and sampling is tentatively scheduled to begin the week of August 23rd. |
| 6/29/21 | DEP Central Laboratory certified DOH private supply well results | DOH | DWRMWELL-2021-06-17-05. Four nearby supply well sample results. One result was above the HAL for combined PFOA and PFOS. |
| 8/2/21 | Trip Report – Site Assessment (Surface Water and DPT Screen Point Sampling) | State Contractor | Observed a private utility locate. Collected 5 surface water samples. Screen point groundwater samples were collected at 10 ft intervals between 16 and 120 ft BLS (11 sample intervals per boring) at 7 locations. Screen point groundwater samples were collected at 10 ft intervals between 16 and 100 ft BLS (9 sample intervals per boring) at 3 locations. Screen point groundwater samples were collected at 10 ft intervals between 126 and 150 ft BLS (3 sample intervals per boring) at 2 locations. Samples were collected from screen point samplers using high-density polyethylene tubing and a peristaltic pump. Staged 14, 55-gallon drums containing liquid investigation-derived waste in a designated area approved by FIRSC personnel. Collected horizontal coordinates of each sampling location using a global positioning system. |
| 8/2/21 | Laboratory analytical results released. SIS-2021-07-20-01 | DEP | The FDEP Central Laboratory released certified analytical results associated with the ongoing supplemental assessment performed in July 2021. Concentrations of combined perfluorooctanoate acid (PFOA) and perfluorooctane sulfonate (PFOS) in groundwater were above the Health Advisory Level (HAL). |
| 8/4/21 | Laboratory analytical results released. SIS-2021-07-23-01 | DEP | The FDEP Central Laboratory released certified analytical results associated with the ongoing supplemental assessment performed in July 2021. Concentrations of combined perfluorooctanoate acid (PFOA) and perfluorooctane sulfonate (PFOS) in groundwater were above the Health Advisory Level (HAL). |
| 8/11/21 | Laboratory analytical results released. SIS-2021-07-19-01 | DEP | The FDEP Central Laboratory released certified analytical results associated with the ongoing supplemental assessment performed in July 2021. Concentrations of combined perfluorooctanoate acid (PFOA) and perfluorooctane sulfonate (PFOS) in groundwater were above the Health Advisory Level (HAL). |
| 8/16/21 | Laboratory analytical results released. SIS-2021-08-02-01 | DEP | The FDEP Central Laboratory released certified analytical results associated with the ongoing supplemental assessment performed in July 2021. Concentrations of combined perfluorooctanoate acid (PFOA) and perfluorooctane sulfonate (PFOS) in groundwater were above the Health Advisory Level (HAL). |
| 9/27/21 | DEP Central Laboratory certified DOH private supply well results | DOH | DWRMWELL-2021-09-09-05. Eleven nearby supply well sample results. Two results were above the HAL for combined PFOA and PFOS. |
| 9/27/21 | DEP Central Laboratory certified DOH private supply well results | DOH | DWRMWELL-2021-09-10-06. Six nearby supply well sample results. Two results were above the HAL for combined PFOA and PFOS. |
| 10/1/21 | Laboratory analytical results released. SIS-2021-09-08-02 | DEP | The FDEP Central Laboratory released certified analytical results associated with the ongoing supplemental assessment performed in September 2021. Concentrations of combined perfluorooctanoate acid (PFOA) and perfluorooctane sulfonate (PFOS) in groundwater were above the Health Advisory Level (HAL). |